## Casting from Meta Quest

Casting allows you to display what the user sees inside the headset on another device such as a phone, computer, or TV. Before starting, make sure that all devices are connected to the same Wi-Fi network and, if applicable, that they are logged into the same Meta account.

## Contents

- headset-preparation

- Set up the Chromecast

- Cast to a TV using Google Chromecast

- Tips

## Set up the Chromecast

- Plug the Chromecast into an HDMI port on the TV.

- Connect the Chromecast’s USB power cable (either to the TV’s USB port or a wall outlet).

- Turn on the TV and select the HDMI input where the Chromecast is connected.

- Open the Google Home app on your phone.

- Tap “+ → Set up device → New device” and follow the instructions:

  - Allow Bluetooth and location access.

  - Confirm the code shown on the TV.

  - Connect Chromecast to your Wi-Fi network.

- Wait for the device to update and complete setup.

- Once done, your TV will show the Chromecast home screen. .

### Cast to a TV using Google Chromecast

Requirements

- A Google Chromecast device (or a smart TV with Chromecast built-in).

- The Google Home app installed on your phone.

- All devices on the same Wi-Fi network.

#### Cast from the Meta Quest headset

- Put on your Meta Quest 2 or 3 headset.

- Press the Meta (Oculus) button on the right controller to open the Universal Menu.

- Select the clock on the left-hand side to open Quick Settings.

- Tap Cast.

- In the list of devices, select your Chromecast or TV.

- Select Next, and the TV will begin showing the headset display.

To stop casting

- Open Quick Settings → Cast → Stop Casting.

Cast to tv

### Tips

- If casting doesn’t appear, verify all devices are on the same Wi-Fi network and frequency band (2.4GHz or 5GHz).

- Restart your headset, Chromecast, or router if the device list doesn’t update.

- For best performance, keep all devices close to the Wi-Fi router.

- You can also cast to the Meta Quest Link app on PC if it’s connected to the headset.